Flight Test I

Drag Polar / Power Required

For

The KOPP BD-4 N375JK “Miss Daisy”

By LT Kenneth G. Kopp

CO-Builder/Owner of the Kopp BD-4 “Miss Daisy”

Table of Contents

List of figures	3

Introduction	4

Kopp BD-4	5

Wings	5

Fuselage	5

Empennage	6

Mission	6

Part I - Instrument Position Errors	7

Vpc Determination	9

Vpc  Flight Test	11

Part II – Drag Polar / Power Required	17

Flight Test Procedure	18

Drag Polar / Power Required Flight Test	19

Flight Safety	19

Data Analysis	27

Drag Polar	27

Power-required	34

Summary and Conclusions	44

Appendix A	46

Appendix B	47

## List of figures

Figure 1 Airspeed Position Error Plot	12

Figure 2 Vcas vs. Vias (mph)	13

Figure 3 Ps	14

Figure 4 Hpc Plot	15

Figure 5 Propeller Thrust vs. Airframe Thrust Required	26

Figure 6 Drag Polar	28

Figure 7 Cd vs. Cl^2	30

Figure 8 Calculated Drag Polar	31

Figure 9 Cl / Cd	33

Figure 10 THP vs Vtas (mph)	35

Figure 11 Normalized Power Required	37

Figure 12 Normalized Power Required (curve fit)	38

Figure 13 Power Curve	40

Figure 14 Normalized Drag Polar	41

Figure 15 Prop efficiency	47

LIST OF TABLES

Table 1Kopp BD-4 Specifications	6

Table 2 Calibrated Airspeed Data	11

Table 3 Crew and altitude assignments	19

Table 4 Flight Responsibilities	19

Table 5. 3000 ft Data	20

Table 6 Data Reduction for 3000 feet PA	22

Table 7 Error Corrections for 3000 ft	24

Table 8 Cdo and e	30

Table 9. Max Lift / Drag	32

Table 10 Minimum Power-required	35

Table 11Cdo and e comparisons	40

Table 12 Performance Table	42

Table 13 Summary	44

Table 14 7500 ft Data and Reduction	46

Table 15 Instrument Corrections for 7500 ft	47

# Introduction

The purpose of this report is to formally release findings and conclusions conducted during initial flight test of the experimental homebuilt Kopp BD-4 depicted on the title page.  This is the first of an extensive series of planned flight tests and reports to be generated while investigating the flight characteristics and handling qualities of this airplane.  Specific objectives include:

- Determine measurement accuracies (Vpc & Hpc)

- Determine the Drag Polar (CL vs. CD)

- Develop Power Required Curves

- Estimate (L/D)max

- Determine minimum power required, maximum range and endurance airspeeds.

The test was conducted in two parts.  Part one consisted of instrument position error determination using the course-over-ground method.  Part two consisted of drag polar and power required data collection conducted at  different altitudes and gross weights.  Results of each test were tabulated and reduced using Microsoft Excel and MATLAB 5.03.  Plots of all pertinent data along with the data itself is included within this report.

# Kopp BD-4

The Kopp BD-4 is a single engine, 4 place, and IFR capable homebuilt airplane. It is equipped with a Lycoming O-360-A1A 180 HP horizontally opposed, direct drive, normally aspirated engine turning a 74” Hartzell 7666-2 constant speed propeller.

## Wings

The BD-4 has a cantilever high wing with a 64-415 modified airfoil.  A plain flap of  71% span and cord of 15% MAC can deflect from 0-30.  Ailerons of the sealed configuration, also have a chord of 15% MAC and are deflected differentially by 1” diameter torque tubes.  The unique tubular spar and metal-to-metal bonding used in the wings kept costs of construction and maintenance low, weight light and construction easy.  Three components comprise the entire cantilever spar design; the center section and two slightly larger wing tubes which are all bolted together with four AN4 bolts ( not so jokingly referred to as Jesus bolts).

## Fuselage

The all-metal fuselage was fabricated entirely of simple flat aluminum gussets and varying length angles of different dimension.  The entire assembly is bolted together “erector set style” using the highest quality AN hardware.  .020” 2024 T3 aluminum skin is bonded and blind riveted to the structure and together form a sturdy, dependable airframe rated to a limit load of +-6 g’s.

## Empennage

The horizontal tail is of the “all-flying” variety found on many Piper airplanes.  The stabilator consists of a single tubular spar and several rib sections formed into a 63-009 airfoil.  The vertical tail is of similar construction.

Table 1  below is a detailed listing of all Kopp BD-4 specifications..

Table 1Kopp BD-4 Specifications

| #### Wing Span | 25.6 ft | Cabin Width | 42” |
| --- | --- | --- | --- |
| Wing Chord | 4 ft | Cabin length | 89” |
| Wing Area | 102.33 ft2 | Cabin height | 41” |
| Aspect Ratio | 6.4 | Fuel Capacity | 60 gal |
| Aileron Area | 3.5 ft2 | Elevator Def up | 15 |
| Flap Area | 8 ft2 | Elevator Def down | 6 |
| Flap Span | 71% | Trim Tab Up | 18 |
| Aileron Defl Up | 25 | Trim Tab Down | 10 |
| Aileron Defl Down | 17 | Rudder Deflection | +- 25 |
| Length | 21.4 ft | Flap Deflection | 0-30 |
| Horizontal Stab Span | 7.3 ft | Max Gross Weight | 2200 |
| Horizontal Chord | 3 ft | Empty Weight | 1412 lbs |
| Horizontal Stab Area | 21.9 ft2 | Useful Load | 788 lbs |
| Horizontal Stab AR | 2.4 | Wing Loading | 21.5 lbs/ft2 |
| Vertical Stab Area | 12 ft2 | Power Loading | 12 lbs/BHP |

## Mission

The Kopp BD-4’s designed mission is that of medium range cross-country cruiser and general recreational aircraft.  The main focus of these flight tests will be to determine how its performance aids or deters fulfillment of its designed mission.

# Part I - Instrument Position Errors

Prior to any serious flight test it is necessary to determine  accuracies of all measurements taken during data collection.  Determination of and documenting these accuracy leads to a more quantitative analysis of the airplanes performance and helps ensure more consistent and meaningful results.  Because the performance of the Kopp BD-4 falls well below the accepted speed threshold of M=.3, where effects of compressibility become significant, all calculations will assume incompressible flow.  The  instrumentation used for this test consists of the installed primary flight performance instruments, which include airspeed (a/s), altimeter (alt), vertical speed indicator (vsi) and the outside air temperature gauge (oat).  With the exception of OAT the most common source of error for the remaining instruments are those related to the static pressure port position, hence the term static position error is used.  Because the A/S, Alt and VSI indicators all operate by measuring  static ambient pressure via the static port, differences between actual ambient pressure and the pressure sensed by the instrument results in an error indicated by the individual instrument.  Unfortunately it is very difficult, if not impossible, to position the static port such that no errors in measurement of static ambient pressures, Ps, are introduced during all flight regimes.  Because the static port is usually located along the fuselage, sensed static pressure, P`s, will vary as fuselage boundary layer conditions vary.  Therefore; errors introduced because of static port position also vary with flow conditions.  For this reason the difference between Ps  and  P`s or Ps,  must be calculated throughout the airplanes complete range of airspeeds and configurations (flaps up and down, gear up and down, etc…).  Additionally each instrument has an internal error called instrument error that must be determined by the manufacturer or tested in a calibrated laboratory environment.  To achieve the highest degree of accuracy when reducing the data collected a correction factor for each error source must be applied to the recorded (indicated) data.  Ps is difficult to measure directly without additional costly equipment.  Instead it is much simpler to determine the position error of a single indicator, such as A/S, and mathematically relate this to the others to determine their errors as well.  Once individual indicator position errors are determined, Ps can be solved analytically. The two primary indicator correction factors are Vpc and Hpc for the A/S and Altimeters respectively. Once these factors are obtained the following sequence is used to correct the data:

Vi		Indicated airspeed (as read on the gauge)

+Vic		instrument correction (from lab)

=Vic                  	Indicated corrected airspeed

+Vpc		static position correction

= Vcal		Calibrated Airspeed

+ Vcomp	compressibility correction (for M>.3)

= Ve		Equivalent airspeed

/σ		correction for density at flight condition with reference to standard sea level density

= V		True Airspeed, actual flight speed

For altitude corrections:

Hi		indicated pressure altitude (set to 29.92)

+Hic 		instrument correction (from lab)

= Hic		Indicated Pressure Altitude corrected for inst. error.

+Hpc		Position Error correction for static position error.

= Hc		Calibrated Pressure Altitude.

## Vpc Determination

The ground-course method was chosen to determine Vpc for the Kopp BD-4 due to its simplicity and because it requires no additional support equipment unlike other commonly used methods such as the tower-fly-by and trailing-bomb techniques. The ground-course flight procedure and data reduction is outlined below:

- Pick two easily identifiable (from the air)  points 4-6 sm apart. Preferably along a stretch of straight road or highway as represented below:

wind

A

B

course

- Fly constant course between points A & B at 5 mph intervals from Vs1+5 to Vmax.

- Record Hi  (29.92), Vi, Ti and the time between points in both directions for each Vi.

- Maintain altitude and heading during each run.  Allow airplane to drift with cross wind.

- Determine Vground for each run by dividing the known distance between points A & B by the elapsed time to transit the two points.  The result is the actual speed over the ground flown during the leg.

- Determine true velocity, Vtas,  from Vtas=Vg1+Vg22.  Vtas =  Vground  in still air, therefore averaging groundspeeds has the effect of eliminating (or minimizing) the effect of wind when timing the runs, thereby providing a means for determination of actual true airspeed.   To fully minimize effects of wind, it is advisable to fly in the early morning or late evening when winds are generally much calmer. Also, a direct crosswind is better than a pure head or tailwind.

- Using the equation of state ρ=PRTi, determine the air density at Hp.  Pressure, P is determined from Hp using standard atmosphere tables, Ti is the outside ambient air temperature read on the OAT gauge.  At low mach numbers correction of  Ti  for stagnation effects results in minimal accuracy gain.

- Solve for Ve by using the formula Ve=VtasPρ0RT

- Because M<.3 it is assumed Ve = Vc.

- Vpc= Vc - Vi

## Vpc  Flight Test

Flight test to determine Vpc was conducted 11 November, 1999 over a 4.17 statute mile section of hwy 101 between Salinas and Soledad, California.  During preflight chart study two bridges located at prominent intersections along the course to be flown were chosen as start and stop points for each run. Each run was conducted in the clean configuration (flaps up).  A Magellan 3000XL handheld GPS was used to refine the charted distance during flight.  The test was conducted under severe clear VFR (visual flight rules) conditions at 10:00 a.m.. Surface winds reported from Salinas automated weather service at the time of test were 120 at 4 knots. The flight was conducted single piloted and flown according to the procedures outlined previously.  After each run a button-hook maneuver was executed to reverse course to arrive on altitude and  airspeed prior to the start point of the next run.  If the start point was reached prior to attainment of the target altitude and airspeed the run was aborted and another button-hook performed.  Table 2 below lists all data and results of this test.

Table 2 Calibrated Airspeed Data

| Date | Date |  |  | Weight | Distance | ρ sea | P1000 | ρ 1000ft | sigma |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11/11/99 | 11/11/99 |  |  | 1800 | 4.17 | 0.00237 | 2040.9 | 0.00229 | 0.96567 |  |  |  |  |
| Run 1 |  |  |  |  | Run 2 |  |  |  |  | Data Reduction |  |  |  |
| Vias(mph) | time (sec) | alt | oat | Vias(mph) | time (sec) | alt | oat | V1g | V2g | Vtas(mph) | Ve(mph) | Vias ave | Vpc |
| 162 | 90.77 | 1000 | 59 | 160 | 97.57 | 1005 | 59 | 165.39 | 153.86 | 159.62 | 156.86 | 161.00 | -4.14 |
| 155 | 94.81 | 1010 | 59 | 155 | 101.32 | 1000 | 59 | 158.34 | 148.16 | 153.25 | 150.60 | 155.00 | -4.40 |
| 148 | 99.18 | 1000 | 59 | 148 | 106.17 | 990 | 59 | 151.36 | 141.40 | 146.38 | 143.84 | 148.00 | -4.16 |
| 142 | 103.56 | 1020 | 59 | 140 | 111.51 | 1000 | 59 | 144.96 | 134.62 | 139.79 | 137.37 | 141.00 | -3.63 |
| 138 | 107.53 | 1000 | 59 | 138 | 111 | 1000 | 59 | 139.61 | 135.24 | 137.43 | 135.05 | 138.00 | -2.95 |
| 129 | 115.82 | 1000 | 59 | 129 | 118.93 | 1000 | 59 | 129.61 | 126.23 | 127.92 | 125.71 | 129.00 | -3.29 |
| 115 | 129.85 | 1000 | 59 | 115 | 129.36 | 1000 | 59 | 115.61 | 116.05 | 115.83 | 113.82 | 115.00 | -1.18 |
| 103 | 142.85 | 1020 | 59 | 103 | 141.17 | 1010 | 59 | 105.09 | 106.34 | 105.71 | 103.88 | 103.00 | 0.88 |
| 96 | 151.20 | 990 | 59 | 96 | 149.37 | 1010 | 59 | 99.29 | 100.50 | 99.89 | 98.16 | 96.00 | 2.16 |
| 80 | 173.69 | 1000 | 59 | 80 | 173.19 | 1000 | 59 | 86.43 | 86.68 | 86.55 | 85.06 | 80.00 | 5.06 |
| 75 | 188.64 | 1000 | 59 | 75 | 179.6 | 1000 | 59 | 79.58 | 83.59 | 81.58 | 80.17 | 75.00 | 5.17 |

Vpc obtained in the last column of table 1  along with a fifth order polynomial curve fit of the data is plotted below in figure 1.

Figure 1 Airspeed Position Error Plot

mph

Using MATLAB to determine the coefficients of the polynomial fit, the equation for Vpc(Vias) is:

ΔVpc=−5.74×10−9Vias5+3.62×10−6Vias4−9.05×10−4Vias3+.113Vias2−7.342Vias+200.27

This equation is used to analytically determine  Vpc for all values of Vi throughout the clean configuration envelope.

For  use in the cockpit a more useful tool is a plot of Vcas vs. Vias as shown below in figure 2.

Figure 2 Vcas vs. Vias (mph)

To obtain an analytic solution for Vcas at any Vias a first order polynomial fit was used to generate the following equation:

Vcas=.8798Vias+15.0543

With this equation for Vpc it is possible to determine Hpc  and Ps from the following relationships derived in the NPS,  AA4323 Flight Test Engineering Class notes:

ΔHpc=Vicσstdgο1+γ−12Vicaο21γ−1ΔVpc , where Vic and Vpc must be in ft/sec.

and

ΔPs=−γpοaο21+γ−12Vicaο221γ−1VicΔVpc

Close examination of the above two equations reveals  Ps  to be insensitive to altitude and atmospheric conditions while Hpc is affected by changes in altitude, as would be expected in an altimeter!  Therefore, Hpc must be determined for each altitude at which a flight test was conducted.

By substituting the 5th order polynomial fit for Vpc into the above two equations and iterating throughout the full range of airspeeds (Vi), the following plots and equations result:

Figure 3 Ps

Where Ps can be represented by:

ΔPs=−1.27×10−6Vic3+4.75×10−4Vic2−.0034Vic−6.704

using a 3rd order polynomial fit of the plot in figure 3 above.

Iterating altitude from sea level to 10,000 pressure altitude results in Hpc curves as shown below in figure 4.

Figure 4 Hpc Plot

Where the Hpc is also represented also by a 3rd order polynomial fit equal to:

ΔHpc=1.708×10−5Vic3−.0064Vic2+.0459Vic+89.9893@ 1000 ft Hi

Otherwise for the general solution :

Hpc(σstd,Vic)= ΔHpc=Vicσstdgο1+γ−12Vicaο21γ−1ΔVpc

where

ΔVpc=−5.74×10−9Vias5+3.62×10−6Vias4−9.05×10−4Vias3+.113Vias2−7.342Vias+200.27

and

σstdρstdρo

In truth the values Vpc, Hpc and Ps for this airplane also include the instrumentation errors as data is not available for the errors of the instruments themselves.  The important point however, is to determine the total system error and apply a correction to any further data collected.

# Part II – Drag Polar / Power Required

In this section the findings from flight testing to determine the drag polar and power required for the Kopp BD-4 will be reported.  Determination of an accurate drag polar enables the calculation of many vital performance metrics such as (L/D)max, Cdo, and the Oswald Span Efficiency Factor, e.  Determination of power required curves provides both a graphical and analytic method for determination of other important parameters such as maximum range and endurance airspeed and minimum power required.

An aircraft drag polar is simply a plot of lift vs. drag or Cl vs. Cd for a particular configuration and is a measure of the aerodynamic efficiency of the complete aircraft independent of the installed propulsion source (to the extent the propulsion configuration contributes to added aircraft drag).  Lift is the easiest quantity to determine since in level flight the total lift equals the gross weight of the aircraft at that instant in time.  Drag however is quite difficult to measure with any accuracy in-flight without  additional equipment.  Fortunately, using proprietary software on loan from Hartzell Propeller Inc.  determination of the 7666-2 blade efficiency and thrust generated was greatly simplified. To use this software the relationship of Vtas for given engine power settings, which is a combination of manifold pressure and propeller rpm, must be determined. Altitude and temperature are important inputs as well aircraft weight at the time of testing.  This requires an accurate fuel-burn chart for calculation of  weight as a function of time for each data point. The Lycoming O-360 operator’s manual includes both engine power  and fuel-burn charts for use in computing the necessary information.  Copies of these charts can be found in the appendix.    Determination of power required as a function of airspeed is obtained using the data collected for the drag polar.

## Flight Test Procedure

There are two basic methods for drag-polar data collection, the constant-airspeed and constant-altitude method.  In the constant airspeed method the pilot flies a designated airspeed while power is adjusted as required to maintain altitude at that airspeed.  Power, Hi, Ti and Vi,  are recorded after all parameters have stabilized. This process is repeated over the full range of airspeeds in the designated configuration.  Conversely the constant-altitude method requires the pilot to establish an altitude, set  power to a desired level and adjust pitch attitude to maintain  altitude. Once stabilized, the information is recorded and the next power setting adjusted.  This process is repeated throughout a range of power settings.  The constant-altitude method is beneficial at higher airspeeds because a power schedule can be developed and tabulated during pre-flight planning for organized data collection during flight. Additionally it is much easier to set a specified power setting and record the resulting airspeed than it is to fly an exact airspeed and determine the power setting.  This is largely due to the size and scaling of the manifold pressure and rpm instrument face used for power determination.  Because priory information of minimum power required for this particular airframe is not known, it is necessary to revert to the constant-airspeed method at lower airspeeds. To help ensure accurate data two runs are conducted for each airspeed / power combination.  The data is averaged over both runs to arrive at one set of data for each altitude and weight of interest.

## Drag Polar / Power Required Flight Test

This test was conducted in the Kopp BD-4 on 27 July, 2000 departing from Monterey Peninsula Airport (MRY) at 10:11 am.  Conditions at take-off were:

Wind:	290/8

Alt:	30.04

#### Sky Clear

Rwy:	28R

It was determined during pre-flight planning that two separate runs would be made at different gross weights and altitudes.  Crew  and altitude assignments were as follows:

Table 3 Crew and altitude assignments

| #### Crew | Altitude | Gross Weight (approx) |
| --- | --- | --- |
| LT Ken Kopp / Maj. Jim Hawkins | 3000 ft | 1950 lbs |
| LT Ken Kopp / LT Anthony Fortesque | 7500 ft | 2150 lbs |

The test area was restricted to the Salinas Valley from Salinas to 15 miles South East of King City.  Crew coordination and a thorough test procedures briefing preceded each flight.  Data collection sheets were developed, printed and discussed in detail prior to flight as well. Specific responsibilities were delegated as follows:

Table 4 Flight Responsibilities

| Responsibility | Pilot at the Controls | Pilot Not at the Controls |
| --- | --- | --- |
| ### Flight Safety | Primary | Secondary |
| Airwork | Primary |  |
| Test Procedure |  | Primary |
| Data Recording |  | Primary |
| Communications | Primary | Secondary |
| Navigation | Secondary | Primary |
| Visual Lookout | Secondary | Primary |
| Emergencies | Primary | Secondary |

#### ATC flight following was utilized to the maximum extent possible to aid in collision

#### avoidance.  King City and Salinas Muni were designated primary diverts in the event an emergency due to mechanical failure or weather occurred.

Each pilot was responsible for one run.  At the completion of a run a control swap was accomplished and the second run completed.  The results of the test conducted at 3000 feet pressure altitude are listed in table 5 below. 7000 feet data is included in the appendix.

Table 5. 3000 ft Data

| Flight Test Data Sheet |  |  |  |  | Power Required |  |  |  |  | N375JK Kopp BD-4 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| st fuel | G W | Wind | Alt | Temp | Srt T | T/O T | C pwr | lvl T | trans pwr | C BR | GW | A br | Delt Time | final gw | Ave GW | Ts |
| 33 | 1991.4 | 290/8 | 30.04 | 16 | 10:00 | 10:11 | 160 | 10:16 | 164 | 13 | 1984 | 7.8 | 63 | 1928.34 | 1956 | 50 |
| Run #1 | Ken |  |  |  | Test Data |  |  |  |  | Run #2 | Jim |  |  | Test Data |  |  |
| MP | RPM | IAS | PA | OAT | Time | Ind HP | BR | GW | MP | RPM | IAS | PA | OAT | TIme | Ind HP | GW |
| 26.5 | 2700 | 154 | 2995 | 69 | 2.0 | 163 | 13 | 1981 | 26.5 | 2700 | 154 | 3000 | 69 | 1.0 | 163 | 1927.0 |
| 26 | 2600 | 150 | 2995 | 68 | 2.0 | 159 | 13 | 1978 | 26 | 2600 | 150 | 3000 | 69 | 1.0 | 159 | 1928.4 |
| 25.5 | 2550 | 148 | 3000 | 68 | 1.0 | 152 | 11.5 | 1977 | 25.5 | 2550 | 146 | 3000 | 69 | 2.0 | 152 | 1929.9 |
| 25 | 2500 | 143 | 2995 | 68 | 1.0 | 146 | 11.5 | 1976 | 25 | 2500 | 142 | 3010 | 69 | 1.0 | 146 | 1932.5 |
| 24.5 | 2450 | 137 | 3000 | 66 | 2.0 | 140 | 11.5 | 1973 | 24.5 | 2450 | 140 | 3020 | 69 | 1.0 | 140 | 1933.8 |
| 24 | 2400 | 135 | 2998 | 67 | 2.0 | 132 | 8.8 | 1971 | 24 | 2400 | 135 | 3020 | 69 | 1.0 | 132 | 1935.1 |
| 23.5 | 2350 | 131 | 3000 | 68 | 1.0 | 124 | 8.8 | 1970 | 23.5 | 2350 | 131 | 3020 | 69 | 1.0 | 124 | 1936.1 |
| 23 | 2300 | 129 | 3000 | 68 | 1.0 | 120 | 8.8 | 1969 | 23 | 2300 | 129 | 3000 | 69 | 1.0 | 120 | 1937.1 |
| 22.5 | 2250 | 122 | 3000 | 68 | 1.0 | 114 | 7.5 | 1968 | 22.5 | 2250 | 125 | 3000 | 69 | 1.0 | 114 | 1938.1 |
| 22 | 2200 | 119 | 3000 | 68 | 1.0 | 108 | 7.5 | 1967 | 22 | 2200 | 122 | 3020 | 69 | 1.0 | 108 | 1939.0 |
| 21.5 | 2200 | 115 | 3000 | 70 | 2.0 | 104 | 7.5 | 1966 | 21.5 | 2200 | 120 | 3000 | 69 | 1.0 | 104 | 1939.8 |
| 21 | 2200 | 116 | 3000 | 69 | 2.0 | 100 | 6.3 | 1964 | 21 | 2200 | 119 | 2990 | 69 | 1.0 | 100 | 1940.7 |
| 20.5 | 2200 | 113 | 3000 | 69 | 2.0 | 96 | 6.3 | 1963 | 20.5 | 2200 | 115 | 3000 | 69 | 1.0 | 96 | 1941.4 |
| 20 | 2200 | 112 | 2995 | 69 | 1.0 | 92 | 6.3 | 1962 | 20 | 2200 | 110 | 3000 | 69 | 1.0 | 92 | 1942.1 |
| 19.5 | 2200 | 109 | 2995 | 69 | 1.0 | 89 | 6.3 | 1961 | 19.5 | 2200 | 104 | 3010 | 69 | 1.0 | 89 | 1942.8 |
| 19 | 2200 | 104 | 3000 | 69 | 2.0 | 86 | 6.3 | 1960 | 19 | 2200 | 95 | 3050 | 69 | 1.0 | 86 | 1943.5 |
| 18.5 | 2200 | 100 | 2995 | 69 | 1.0 | 82 | 5.5 | 1959 | 18.5 | 2200 | 94 | 3000 | 69 | 1.0 | 82 | 1944.2 |
| 18 | 2200 | 94 | 3000 | 69 | 3.0 | 78 | 5.5 | 1957 | 18 | 2200 | 90 | 2970 | 69 | 2.0 | 78 | 1944.9 |
| 18 | 2200 | 90 | 2995 | 68 | 2.0 | 78 | 5.5 | 1956 | 17 | 2200 | 88 | 3000 | 69 | 1.0 | 72 | 1946.1 |
| 17.5 | 2200 | 85 | 3000 | 68 | 2.0 | 74 | 5.5 | 1955 | 19 | 2200 | 76 | 3050 | 69 | 1.0 | 86 | 1946.7 |
| 17 | 2200 | 78 | 3000 | 69 | 1.0 | 72 | 5.5 | 1954 | 18 | 2200 | 70 | 2925 | 69 | 2.0 | 78 | 1947.4 |
| 17.5 | 2200 | 74 | 3050 | 68 | 3.0 | 74 | 5.5 | 1952 |  |  |  |  |  |  |  |  |
| 18 | 2200 | 70 | 3000 | 70 | 6.0 | 78 | 5.5 | 1949 |  |  |  |  |  |  |  |  |

The top row of the table consists of starting weight, starting fuel, basic weather information, engine start time, take off time, climb power and level off time.  These values are used to determine the fuel burned during t/o, climb and transit to the working area in order to calculate an accurate test start weight.  The first six columns for each run are recorded data; manifold pressure, propeller rpm, indicated airspeed, pressure altitude, outside air temperature and the elapsed time between power changes.  Engine power was determined through use of the manufacturers supplied engine power chart provided in the appendix.  With engine HP recorded the fuel-burn chart, also included in the appendix, was entered and the corresponding value placed on the data sheet.  A running reduction in aircraft gross weight was calculated according to the following relationship:

GW=GWpreviouslbsΔtimemin60minhrburnrategalhr⋅6.8lbsgal

The excel spreadsheet shown in table 6 automatically links raw  data from table 5 and calculates results for export to MATLAB for further analysis and plotting.

Table 6 Data Reduction for 3000 feet PA

| Columns 1-4 are linked cells and are self explanatory as is column 5. Column 6 is the average gross weight of the aircraft at the time each data point was collected. Vcas in column 7 is derived from the curve fit data at the bottom of the table. The curve fit was obtained as discussed in the previous section. Vtas and Mach number are calculated for entry into the propeller thrust and efficiency software. Indicated HP was obtained from the engine chart . Because the Lycoming O-360-A1A is normally aspirated (fancy way of saying it uses a carburetor), the power generated is a function of the ratio of standard atmospheric temperature (at a specific pressure altitude) and the inlet temperature. For our purpose we will assume the OAT measurement of static ambient temperature is equal to the inlet temperature. In fact this may not be a reasonable assumption as inlet air passes many very hot engine components prior to fuel-air atomization. In effect this ratio is a measure of combustion efficiency and is calculated according to the following relationship: ##### Reduced Data for 3000 ft and 1950 lbs |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| MP | RPM | IAS | IAS | Ave IAS | AveGW | Vcas | Vtas | M | Ind HP | Corr HP | n | THP | Thrust |
| 26.5 | 2700 | 154 | 154 | 154 | 1954.03 | 149.54 | 159.26 | 0.207 | 163 | 159.6 | 0.761 | 121.5 | 286 |
| 26 | 2600 | 150 | 150 | 150 | 1953.29 | 146.05 | 155.54 | 0.202 | 159 | 155.7 | 0.771 | 120.0 | 297 |
| 25.5 | 2550 | 148 | 146 | 147 | 1953.38 | 143.43 | 152.75 | 0.199 | 152 | 148.8 | 0.774 | 115.2 | 293 |
| 25 | 2500 | 143 | 142 | 142.5 | 1954.03 | 139.50 | 148.57 | 0.193 | 146 | 143.0 | 0.773 | 110.5 | 295 |
| 24.5 | 2450 | 137 | 140 | 138.5 | 1953.38 | 136.00 | 144.85 | 0.188 | 140 | 137.1 | 0.773 | 106.0 | 295 |
| 24 | 2400 | 135 | 135 | 135 | 1953.03 | 132.95 | 141.59 | 0.184 | 132 | 129.2 | 0.776 | 100.3 | 286 |
| 23.5 | 2350 | 131 | 131 | 131 | 1953.03 | 129.45 | 137.87 | 0.179 | 124 | 121.4 | 0.778 | 94.5 | 277 |
| 23 | 2300 | 129 | 129 | 129 | 1953.03 | 127.71 | 136.01 | 0.177 | 120 | 117.5 | 0.777 | 91.3 | 269 |
| 22.5 | 2250 | 122 | 125 | 123.5 | 1953.10 | 122.90 | 130.89 | 0.170 | 114 | 111.6 | 0.772 | 86.2 | 262 |
| 22 | 2200 | 119 | 122 | 120.5 | 1953.10 | 120.28 | 128.10 | 0.167 | 108 | 105.7 | 0.773 | 81.7 | 252 |
| 21.5 | 2200 | 115 | 120 | 117.5 | 1952.68 | 117.66 | 125.31 | 0.163 | 104 | 101.8 | 0.771 | 78.5 | 253 |
| 21 | 2200 | 116 | 119 | 117.5 | 1952.39 | 117.66 | 125.31 | 0.163 | 100 | 97.9 | 0.778 | 76.2 | 242 |
| 20.5 | 2200 | 113 | 115 | 114 | 1952.03 | 114.61 | 122.06 | 0.159 | 96 | 94.0 | 0.783 | 73.6 | 230 |
| 20 | 2200 | 112 | 110 | 111 | 1952.03 | 111.99 | 119.27 | 0.155 | 92 | 90.1 | 0.784 | 70.6 | 223 |
| 19.5 | 2200 | 109 | 104 | 106.5 | 1952.03 | 108.06 | 115.08 | 0.150 | 89 | 87.1 | 0.781 | 68.1 | 222 |
| 19 | 2200 | 104 | 95 | 99.5 | 1951.68 | 101.94 | 108.57 | 0.141 | 86 | 84.2 | 0.772 | 65.0 | 225 |
| 18.5 | 2200 | 100 | 94 | 97 | 1951.72 | 99.76 | 106.25 | 0.138 | 82 | 80.3 | 0.771 | 61.9 | 219 |
| 18 | 2200 | 94 | 90 | 92 | 1951.10 | 95.39 | 101.60 | 0.132 | 78 | 76.4 | 0.776 | 59.3 | 217 |
| 18 | 2200 | 90 | 90 | 90 | 1951.10 | 93.65 | 99.74 | 0.130 | 78 | 76.4 | 0.763 | 58.3 | 219 |
| 17.5 | 2200 | 85 |  | 85 | 1950.79 | 89.28 | 95.09 | 0.124 | 74 | 72.4 | 0.758 | 54.9 | 216 |
| 17 | 2200 | 78 |  | 78 | 1950.79 | 83.17 | 88.58 | 0.115 | 72 | 70.5 | 0.742 | 52.3 | 223 |
| 17.5 | 2200 | 74 |  | 74 | 1952.35 | 79.67 | 84.86 | 0.110 | 74 | 72.4 | 0.728 | 52.7 | 235 |
| 18 | 2200 | 70 | 70 | 70 | 1948.61 | 76.18 | 81.13 | 0.106 | 78 | 76.4 | 0.708 | 54.1 | 252 |
|  | Atmospheric Data |  | Temp |  | s sound | CAS Curve Fit |  |  |  |  |  |  |  |
| sigma | rstd | P3000 | Ts | T | a | slope | intercept |  |  |  |  |  |  |
| 0.88161 | 0.00237 | 1896.7 | 48.3 | 69 | 1127.33 | 0.87 | 15.05 |  |  |  |  |  |  |

HPcorrected=HPindicated460.87+Ts460.87+Ti     ,  where Ts and Ti are given in F.

If the altimeter had zero instrument error and it was known the static source was also error free, Ts would correspond to the standard temperature found in the atmosphere tables for the Hi (indicated pressure altitude) flown.  However, as discussed in the previous section it is necessary to apply error corrections to Hi  in order to obtain the true  pressure altitude, Hc,  flown at each test point.  Once Hc is obtained the standard atmosphere tables can be used to retrieve the actual Ts and Ps  through interpolation.  With Ts determined for each data point a more accurate calculation of HPcorrected can be obtained.  Ps is used to calculate actual air density at Hc and Ti through use of the equation of state for a perfect gas as shown below.

ρactual=PsRTi+460.87. Density is a key factor in accurately determining the lift and drag coefficients CL & CD used extensively in performance analysis.

Through application of the equations derived in the previous sections and development of yet another spreadsheet the following corrections and values were obtained in table 7.

Table 7 Error Corrections for 3000 ft

| 3000ft | rho std | sigstd | gama | ao |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0.0022 | 0.918 | 1.4 | 1116.3 |  |  |  |
| Ave Ias | Ave Alt |  Vpc |  Hpc | Hc | Ts | Ps | rho |
| 154 | 2997.50 | -2.44 | -27.55 | 2969.95 | 47.23 | 1899.00 | 0.0020885 |
| 150 | 2997.50 | -2.40 | -26.34 | 2971.16 | 47.22 | 1898.92 | 0.0020884 |
| 147 | 3000.00 | -2.34 | -25.18 | 2974.82 | 47.21 | 1898.66 | 0.0020881 |
| 142.5 | 3002.50 | -2.20 | -23.00 | 2979.50 | 47.19 | 1898.34 | 0.0020878 |
| 138.5 | 3010.00 | -2.03 | -20.60 | 2989.40 | 47.16 | 1897.64 | 0.002087 |
| 135 | 3009.00 | -1.84 | -18.18 | 2990.82 | 47.15 | 1897.54 | 0.0020869 |
| 131 | 3010.00 | -1.57 | -15.06 | 2994.94 | 47.14 | 1897.25 | 0.0020866 |
| 129 | 3000.00 | -1.42 | -13.39 | 2986.61 | 47.17 | 1897.84 | 0.0020872 |
| 123.5 | 3000.00 | -0.94 | -8.48 | 2991.52 | 47.15 | 1897.49 | 0.0020869 |
| 120.5 | 3010.00 | -0.64 | -5.65 | 3004.35 | 47.10 | 1896.60 | 0.0020859 |
| 117.5 | 3000.00 | -0.32 | -2.74 | 2997.26 | 47.13 | 1897.09 | 0.0020864 |
| 117.5 | 2995.00 | -0.32 | -2.74 | 2992.26 | 47.15 | 1897.44 | 0.0020868 |
| 114 | 3000.00 | 0.08 | 0.70 | 3000.70 | 47.12 | 1896.85 | 0.0020862 |
| 111 | 2997.50 | 0.45 | 3.69 | 3001.19 | 47.12 | 1896.82 | 0.0020861 |
| 106.5 | 3002.50 | 1.05 | 8.21 | 3010.71 | 47.08 | 1896.15 | 0.0020854 |
| 99.5 | 3025.00 | 2.11 | 15.30 | 3040.30 | 46.98 | 1894.08 | 0.0020831 |
| 97 | 2997.50 | 2.52 | 17.88 | 3015.38 | 47.07 | 1895.82 | 0.002085 |
| 92 | 2985.00 | 3.45 | 23.14 | 3008.14 | 47.09 | 1896.33 | 0.0020856 |
| 90 | 2997.50 | 3.85 | 25.31 | 3022.81 | 47.04 | 1895.30 | 0.0020845 |
| 85 | 3025.00 | 4.99 | 30.98 | 3055.98 | 46.92 | 1892.98 | 0.0020819 |
| 78 | 2962.50 | 6.99 | 39.76 | 3002.26 | 47.11 | 1896.74 | 0.002086 |
| 74 | 3050.00 | 8.41 | 45.38 | 3095.38 | 46.78 | 1890.22 | 0.0020789 |
| 70 | 3000.00 | 10.09 | 51.54 | 3051.54 | 46.94 | 1893.29 | 0.0020822 |
|  |  |  |  |  | Interpolation Values for 3000 ft |  |  |
|  |  |  |  |  | A | Ts | Ps |
|  |  |  |  |  | 2500 | 509.77 | 1931.9 |
|  |  |  |  |  | 3500 | 506.21 | 1861.9 |

Referring back to table 6, HPcorrected can now more accurately be solved with the calculated values of Ts  listed above.

With Mach number, altitude, rpm, HPcorrected and Ti, Hartzell’s propeller software is used to generate the efficiency and thrust generated by the propeller.  The software contains proprietary performance maps of the 7666-2 blade generated through many hours of ground and flight testing and assumes a .4 blockage factor in thrust calculations. Because Lycoming engines are of the direct drive category, meaning engine crankshaft and propeller are connected directly and turns at the same rate, HPcorrected is equivalent to the more commonly used term Shaft Horsepower or SHP.  Engines equipped with reduction drives must account for less than 100% power delivery to the propeller in calculations of SHP.

With SHP determined and software generated propeller efficiency in hand, it is now possible to determine the actual power delivered to the airframe by the engine / propeller combination.  This power is referred to as Thrust Horsepower or THP and is determined by the simple relationship shown below:

THP=η×SHP  , where η is propeller efficiency and is precisely how values in column 13 of table 6 were determined.  Column 14 lists the thrust calculated by the Hartzell program as a function of Vtas.  The software generated thrust is determined based on a standard fuselage blockage factor of approximately .4. This results in thrust values while  representative of the actual thrust developed by the propeller they are higher than the thrust required by the airframe to maintain level flight.  This is an important distinction if the power required data generated is to remain independent of the propulsive system installed.  The thrust (drag) required by the airframe is calculated then from the equation below.

Thrust=THP×550ft⋅lbssec⋅HPVftsec

A plot of the propeller generated thrust and airframe required thrust clearly illustrates the  effect of the inclusion of blockage factor during thrust calculation.

Figure 5 Propeller Thrust vs. Airframe Thrust Required

As may be expected as velocity increases the effect of fuselage blockage becomes greater as highlighted in the plot.  All remaining calculations will be based on the airframe required thrust calculated according to the equation described on the previous page.

# Data Analysis

## Drag Polar

As discussed previously the drag polar is a measure of an airframes aerodynamic efficiency independent of the installed propulsion system and is generated by plotting Lift vs. Drag.  Because total lift and drag are cumbersome terms it is common to plot the drag polar as a function of the non-dimensional lift and drag coefficients, Cl and Cd according to the following relationships:

Cl=Lift12ρV∞2S , where V∞ is Vtas is ft/sec and S is the total aircraft wing area .

and

Cd=Drag12ρV∞2S, recalling basic Newtonian physics;  in level unaccelerated flight the airplane is in equilibrium, therefore:

Fz=maz=0  and    Fx=max=0    shown pictorially below.

Lift

Weight

Drag

Thrust

Thus,  lift = weight and drag = thrust.  Substitution of these equalities leads to:

Cl=Weight12ρV∞2S and Cd=Thrust12ρV∞2S.

The drag polar can now be plotted from our tabulated data obtained during flight test and as shown in figure 6 below.

Figure 6 Drag Polar

Another method for determining the drag coefficient is provided by finite wing theory, which predicts Cd to be equal to the sum of parasite and induced drag given by the following equation.

Cd=Cdo+Cl2πeAR,  where Cdo is the zero lift drag coefficient, e is  the Oswald Span Efficiency Factor and accounts for non-elliptical shaped wing planforms and the contribution to parasite drag due to lift.  AR is the wing aspect ratio calculated as:

induced drag

parasite drag

AR=b2S ,  where b is  wing span and S is  total wing area.

Determination of  an accurate Cdo is an important step in the design process of all aircraft.  Parasite drag is the major source of drag at high speeds and therefore must be determined early in the design phase for accurate engine sizing, weight determination and performance calculations.  For an airplane still on the design table, detailed methods have been developed by aerospace manufactures to estimate Cdo through elaborate accounting  of parasite drag of all wetted components.  Thankfully the benefit of a fully functional airplane allows us to solve for Cdo and e directly from flight test data.  This also provides a means to double check the theory as an academic exercise.  Recalling the finite wing theory equation for Cd as

CD=Cdo+CL2πeAR,  notice that CD is linear in CL2 !

Cd and Cl have been determined previously from flight test data  and plotted in figure 5 as a drag polar.  All that remains is to plot CD as a function of CL2 and determine the slope and intercept for e and Cdo respectively.  CD vs CL2 is shown in figure 7.

Figure 7 Cd vs. Cl^2

Data for both the 3000 ft 1950 lb and 7500ft 2130 lbs case converge nicely showing that values of e and Cdo are relatively insensitive to atmospheric and weight changes .    Using MATLAB to determine the slope and intercept of these two curves results in:

| Altitude | Oswald Span Efficiency | Cdo |
| --- | --- | --- |
| 3000 ft | .7039 | 0.0440 |
| 7500 ft | .7289 | 0.0433 |

Table 8 Cdo and e

with e and Cdo for each altitude Cd is recalculated according to the finite wing equation and a new drag polar plotted based on the theory as shown below in figure 8.

Figure 8 Calculated Drag Polar

Clearly the theory does an excellent job of determining the drag coefficient from calculated values of Cdo and e.

Recalling the free-body diagram depicted on page 26, from the equilibrium sum of forces during level unaccelerated flight where lift=weight and drag=thrust we can derive an important relationship as follows:

Lift=Weight=q∞SCl    and  where Thrust=Drag=q∞SCd, where q∞=.5ρ∞Vtas2

Therefore:

LiftDrag=WeightThrust=q∞SClq∞SCd=ClCd  , Thus

Thrust=WeightClCd,  from this equation it is apparent that minimum thrust required to maintain level unaccelerated flight occurs when the ratio Cl/Cd is a maximum!

Because values for Cl and Cd have been calculated from the data it is a simple matter of choosing the largest value of the resulting ratio.  Using MATLAB to carry out this calculation results in values of:

Table 9. Max Lift / Drag

| #### Altitude / Weight | Max Cl/Cd | Min Thrust Required |
| --- | --- | --- |
| 3000 ft / 1950 lbs | 8.955 | 217.87 lbs |
| 7500 ft / 2130 lbs | 9.1957 | 229.63 lbs |

Referring back to table 6 for the reduced data at 3000 ft shows the recorded minimum thrust (highlighted in red) to be 222 lbs, thus providing excellent correlation with the theoretical value predicted in table 9 above.  Another useful means for determination of  (L/D)max is simply graphing the ratio and noting the value at which point a maximum occurs as depicted in figure 9 on the next page.

For a given airspeed Cl must increase with gross weight to maintain lift necessary for level flight.  Concurrently as Cl increases Cd increases as a function of Cl2, the net result of which is that (L/D)max remains constant but the airspeed corresponding to (L/D)max increases for an increasing gross weight.  The .24 difference in (L/D)max calculated in table 9 may be attributed to instrument and other errors introduced during data collection.

Figure 9 Cl / Cd

Additionally, the point at which a line drawn from the origin and tangent to the drag polar curve intersects is also the point of (L/D)max.

With the drag polar plot accurately determined many performance factors can now be analyzed as will be discussed in subsequent sections.

## Power-required

For propeller driven aircraft determination of power-required for level unaccelerated flight constitutes a critical step toward performance measurement, prediction and optimization.  The values obtained in this analysis directly impact the operational procedures used by the pilot to fly a particular mission profile. For example, power-required data provides the means to determine maximum range airspeed, a critical performance factor for flights over open water where distances between airfields may be great.  Minimum power-required leads to maximum endurance airspeed. Critical for missions requiring long loiter times, such as patrol and search operations.  The combination of power-required and power available data leads to performance calculations for climb rate, maximum airspeed, altitude performance effects and others.  From a pilots perspective power-required determination is the single most important flight test data for real-time operational use.

In actuality,  power-required data was compiled when the thrust horsepower THP was determined while deriving the drag polar.  All that remains is to plot THP vs. Vtas for a graphical representation of this airplanes power-required as shown in figure 10  on the next page.

Figure 10 THP vs Vtas (mph)

The high altitude, high weight configuration results in a shift of the power-required curve up and to the right.  As altitude increases the minimum power-required increases while (L/D)max  remains constant.  The effect of increased weight is higher minimum power-required and at higher velocity.  Minimum power-required to maintain level unaccelerated flight is simply  the lowest point of each curve.  The values determined from curve fit are:

Table 10 Minimum Power-required

| Altitude | Minimum Thrust Horsepower-required |
| --- | --- |
| 3500 ft 1950 lbs | 52.385 HP |
| 7500 ft 2130 lbs | 60.590 HP |

Although these plots are useful in determining the performance parameters mentioned  at the beginning of this section it would be necessary to derive a curve for each altitude and weight of interest due to their dependence on these parameters as shown above.  To alleviate this tedious dilemma, development of normalized power-required vs. normalized airspeed reduces the data such that the resultant curve is independent of weight and altitude.  In effect, one curve fits all. Once this curve is generated a simple algorithm is used to extrapolate the data and a table of performance parameters for specific weights and altitudes is generated. The normalized parameters Piw & Viw   are power and calibrated (M=.3 and below) airspeed corrected to standard weight sea level conditions.  Where standard weight, Ws is defined as the maximum gross take-off weight for propeller driven aircraft.  For the Kopp BD-4 Ws = 2,200 lbs. Conversion of  Pr & Vtas to their normalized values requires the following equations:

Viw=VtasσWsWt , where Wt is the test weight.

for  Piw  recall

THP=Drag×Vtasft/sec550=15502Wt3Cd2ρ∞SCl3

Piw≡DiwViw550=15002Ws3Cd2ρoSCl3  and therefore Piw=THP5502Wt3Cd2ρ∞SCl3×15002Ws3Cd2ρoSCl3

simplifying the above equation results in: Piw=THPσWs3Wt3

The normalized power required curve can now be plotted and performance parameters determined directly from the plot as shown below on figure  11.

Figure 11 Normalized Power Required

#### Ideally each curve for different weights and altitudes should converge into a single normalized curve.  Unaccounted errors in propeller efficiency, oat instruments, MP and rpm can account for the slightly inconsistent results.  Based on purely qualitative analysis of the conduct of the flight test it was determined the data recorded during the 3000 ft  1950 lbs gross weight run is the more accurate and will therefore be used to derive a 5th order polynomial fit to provide a working equation by which to calculate the remaining parameters.  MATLAB was used to generate the curve fit as follows:

Piw=−2.3044E−10Viw5+1.3425E−7Viw4+3.0956E−5Viw3+3.54377E−3Viw2−.20116Viw+4.58032

Iterating the above equation for a range of Viw from 70 -160 mph results in the curve fit plot of normalized power required shown in figure 12  below.

Figure 12 Normalized Power Required (curve fit)

max endurance airspeed

The next step is to determine the maximum range and endurance normalized airspeeds.  This can be done graphically by drawing a line from the origin to a point of tangency along the Piw curve, the intersection of which corresponds to the maximum range normalized airspeed.  Maximum endurance airspeed occurs at the minimum power required for level unaccelerated flight and is easily determined graphically by selecting the lowest point on the curve or analytically by differentiating the curve fit normalized power curve, equating to zero and solving for velocity.  For propeller driven aircraft maximum range and endurance airspeed occur at (Cl/Cd)max and (Cl3/2/Cd)max.  For use of the normalized curve normalized values of Cl and Cd must be calculated.  Cl  is calculated using Ws and sea level density, ρ∞, throughout the speed range of interest.  Cd is calculated as done previously using the finite wing theory equation for total airplane drag: Cd=Cdo+Cl2πeAR.  To ensure this method is valid values of Cdo and e can be determined by plotting the linear equation:

PiwViw = A1Viw4 + B1 , called the Power Curve, and compared the values determined from the drag polar.  By calculating values for the slope and intercept as A1 and B1 respectively and solving for Cdo and e according to the following relationships:

Cdo=1100A1ρoS  and   e=Ws2275πARSρoB1 total Cd can be determined.

Again ratios Cl/Cd and Cl3/2/Cd can be plotted.  The maximum of each is located and the value of Cl corresponding to each is used to solve for the normalized airspeeds in question.  The desired results are calibrated maximum range and endurance airspeed. With this, the total position error term Vpc can be subtracted to retrieve indicated airspeed as seen by the pilot  in flight. To illustrate this method maximum range airspeed will be determined in this fashion.

The power curve is plotted below:

Figure 13 Power Curve

from the plot and equations listed on the previous page Cdo and e are calculated and compared to the values determined from the drag polar in table 11 below :

| #### Parameters | #### Drag Polar | #### Power Curve |
| --- | --- | --- |
| #### Cdo | #### 0.0440 | #### 0.0425 |
| #### e | #### 0.7031 | #### 0.6507 |

Table 11Cdo and e comparisons

These values show good correlation to those calculated via the drag polar.

#### The normalized drag polar plotted below is used to determine normalized (Cl/Cd)max

and corresponding value of Cl .

Figure 14 Normalized Drag Polar

Cl at (Cl/Cd)max is .744.  Solving for Viw  with this value by

Viw=Ws12ρoS.744=2200.5.00237102.33⋅.744=156.15ft/sec=106.47mph

which corresponds nicely to the tangent drawn to the normalized power required curve in figure 11.  Because Vcas for max range is a function of (L/D)max and remains constant with changes in altitude, variations in Vcas for max range and endurance are dependent on weight changes only. Therefore, maximum range Vcas for any weight is given by:

Vcas=WtWs12Viw   therefore the maximum range airspeed at the first test condition weight of 1950 lbs = 100.23 mph calibrated.

Maximum normalized endurance airspeed is determined by locating the airspeed corresponding to the minimum power required. Referring back to figure 11 for the normalized power required curve reveals 85.5 mph calibrated normalized airspeed gives maximum endurance performance in the Kopp BD-4.  Applying the equation above:

Vcas endurance= 80.49 mph @ 1950 lbs.

The real utility in this approach is the ease in which a performance table can be constructed within a spreadsheet program as displayed below.

Table 12 Performance Table

|  |  |  | Airspeed | Correction |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Vias | 65.0 | 70.0 | 75.0 | 80.0 | 85.0 | 90.0 | 95.0 |
| Vcas | 71.6 | 76.0 | 80.3 | 84.7 | 89.0 | 93.4 | 97.7 |
| Vias | 100.0 | 105.0 | 110.0 | 115.0 | 120.0 | 125.0 | 130.0 |
| Vcas | 102.1 | 106.4 | 110.8 | 115.1 | 119.5 | 123.8 | 128.2 |
| Vias | 135.0 | 140.0 | 145.0 | 150.0 | 155.0 | 160.0 | 165.0 |
| Vcas | 132.5 | 136.9 | 141.2 | 145.6 | 149.9 | 154.3 | 158.6 |
| Vias | 170.0 | 175.0 | 180.0 |  |  |  |  |
| Vcas | 163.0 | 167.3 | 171.7 |  |  |  |  |
|  |  |  | ###### Vcas |  |  |  |  |
| G Weight | 1600 | 1700 | 1800 | 1900 | 2000 | 2100 | 2200 |
| Range | 90.8 | 93.6 | 96.3 | 98.9 | 101.5 | 104.0 | 106.5 |
| Endurance | 72.9 | 75.2 | 77.3 | 79.5 | 81.5 | 83.5 | 85.5 |
|  |  |  | ###### Vias |  |  |  |  |
| G. Weight | 1600 | 1700 | 1800 | 1900 | 2000 | 2100 | 2200 |
| Range | 87.1 | 90.3 | 93.4 | 96.4 | 99.4 | 102.3 | 105.1 |
| Endurance | 66.5 | 69.1 | 71.6 | 74.0 | 76.4 | 78.7 | 81.0 |

The normalized power transformation has utility in an ability to predict the minimum power required for various altitudes and gross weights and can be used to determine the maximum sustainable altitude as illustrated below.

To determine if the Kopp BD-4 can maintain level flight at an altitude of 15,000 feet at max gross weight (2200lbs) use the following relationships:

Piw=59.16hp=THPrequiredσWsWt

σ  @ 15K feet = .6313 therefore;

THPrequired=Piwσ=59.16hp.794=74.45hp

Consultation of the power chart and propeller efficiency software reveals a THPavailable equal to only 71.09 hp.  Therefore, at max gross weight the Kopp BD-4 will not  be capable of maintaining an altitude of 15,000 feet.  A gross weight of 2,000 lbs requires 70.99 hp at 15,000 ft, which should be attainable for standard day conditions. It will be interesting to investigate this prediction during flight test!

## Summary and Conclusions

In this test key instrument calibration correction factors were developed and applied, thereby providing means for more accurate data analysis in subsequent testing.  Applying FAA standards for certified aircraft for a maximum Vpc 0f  +- 6 mph shows  the Kopp BD-4 to be skirting the limit throughout most of its airspeed range.  Further testing of the instrumentation and aircraft are warranted in this matter. Also, an attempt to obtain manufacturer calibration errors may improve the results as well.  Development of the drag polar, power required curves and Cl/Cd provided valuable information immediately usable by the pilot (that’s me!).  A summary table below highlights many of the details resulting from this effort.  Further testing will provide even greater insight into the performance and characteristics of this versatile little airplane.

Table 13 Summary

| #### Altitude / Weight | Max Cl/Cd | Min Thrust Required |
| --- | --- | --- |
| 3000 ft / 1950 lbs | 8.8235 | 217.87 lbs |
| 7500 ft / 2130 lbs | 9.0329 | 229.63 lbs |
| #### Parameters | #### Drag Polar | #### Power Curve |
| #### Cdo | #### 0.0440 | #### 0.0425 |
| #### e | #### 0.7031 | #### 0.6507 |
| Altitude | Minimum Thrust Horsepower Required |  |
| 3500 ft 1950 lbs | 52.33 HP |  |
| 7500 ft 2130 lbs | 60.59 HP |  |
| Standardized | 59.16 HP |  |
| Kopp BD-4 Summary Table Test conducted 27 July, 2000 Data to be added upon further testing |  |  |

Finally, recalling the specific objectives listed in the introduction:

- Determine instrument accuracies (Vpc & Hpc)

- Determine the Drag Polar (CL vs. CD)

- Develop Power Required Curves

- Estimate (L/D)max

- Determine minimum power required, maximum range and endurance airspeeds.

Clearly all objectives were met during this flight test.  Flight Test II will investigate power-available and excess power.  The combination of both sets of data will enable a vast array of performance calculations to be made.

# Appendix A

Table 14 7500 ft Data and Reduction

| Reduced Data for 7500 ft and 2130 lbs |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | run 1 | run 2 |  |  |  |  |  |  |  |  |  |  |
| MP | RPM | IAS | IAS | Ave IAS | Ave GW | Vcas | Vtas | M | HP | Corr HP | n | THP | Thrust |
| 22.7 | 2700 | 129 | 134 | 131.5 | 2128.38 | 129.78 | 150.39 | 0.197 | 142 | 138.0 | 0.754 | 104.1 | 259 |
| 22.7 | 2600 | 130 | 131 | 130.5 | 2125.77 | 128.91 | 149.37 | 0.196 | 139 | 135.1 | 0.764 | 103.2 | 267 |
| 22.7 | 2500 | 126 | 126 | 126 | 2123.28 | 124.98 | 144.82 | 0.190 | 134 | 130.2 | 0.76 | 99.0 | 272 |
| 22.7 | 2450 | 124 | 125 | 124.5 | 2122.28 | 123.67 | 143.30 | 0.188 | 132 | 128.3 | 0.761 | 97.6 | 265 |
| 22.7 | 2400 | 123 | 124 | 123.5 | 2121.28 | 122.80 | 142.29 | 0.187 | 131 | 127.3 | 0.763 | 97.1 | 259 |
| 22.7 | 2300 | 121 | 123 | 122 | 2120.29 | 121.49 | 140.77 | 0.185 | 126 | 122.4 | 0.758 | 92.8 | 247 |
| 22.5 | 2250 | 120 | 120 | 120 | 2117.79 | 119.74 | 138.75 | 0.182 | 124 | 120.5 | 0.75 | 90.4 | 245 |
| 22 | 2200 | 117 | 116 | 116.5 | 2116.52 | 116.68 | 135.21 | 0.177 | 118 | 114.7 | 0.746 | 85.5 | 239 |
| 21.5 | 2200 | 115 | 114 | 114.5 | 2115.67 | 114.94 | 133.18 | 0.175 | 113 | 109.8 | 0.752 | 82.6 | 232 |
| 21 | 2200 | 109 | 110 | 109.5 | 2114.39 | 110.57 | 128.12 | 0.168 | 108 | 104.9 | 0.751 | 78.8 | 230 |
| 20 | 2200 | 103 | 104 | 103.5 | 2113.12 | 105.33 | 122.05 | 0.160 | 102 | 99.1 | 0.749 | 74.2 | 229 |
| 19 | 2200 | 99 |  | 99 | 2111.69 | 101.40 | 117.50 | 0.154 | 95 | 92.3 | 0.745 | 68.8 | 229 |
| 18.5 | 2200 | 90 |  | 90 | 2110.98 | 93.54 | 108.39 | 0.142 | 92 | 89.4 | 0.723 | 64.6 | 238 |
| 19.5 | 2200 | 85 |  | 85 | 2109.91 | 89.17 | 103.33 | 0.136 | 95 | 92.3 | 0.702 | 64.8 | 245 |
| 19 | 2200 | 80 | 76 | 78 | 2108.83 | 83.06 | 96.25 | 0.126 | 92 | 89.4 | 0.677 | 60.5 | 252 |
| 18.5 | 2200 | 75 |  | 75 | 2107.05 | 80.44 | 93.21 | 0.122 | 92 | 89.4 | 0.677 | 60.5 | 252 |
| 21 | 2200 | 70 |  | 70 | 2122.42 | 76.08 | 88.15 | 0.116 | 108 | 104.9 | 0.624 | 65.5 | 279 |
|  | Atmospheric Data |  | Temp |  | Sonic Speed | CAS Curve Fit |  |  |  |  |  |  |  |
| sigma | rstd | p7500 | Ts | T | a | slope | intercept |  |  |  |  |  |  |
| 0.74477 | 0.00237 | 1602.3 | 32 | 60 | 1117.6976 | 0.8733 | 14.9441 |  |  |  |  |  |  |

| Flight Test Data Sheet |  |  | 7500ft |  | Power Required |  |  |  |  | N375JK Kopp BD-4 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GW | fuel | Wind | Alt | T | Srt T | T/O T | C pwr | lvl T | trans pwr | C br | GW | A br | Delt Time | final gw | Ave GW | Ts |
| 2168 | 56 | 290/8 | 30.04 | 17 | 12:30 | 12:39 | 160 | 12:53 | 164 | 13 | 2147 | 8 | 66 | 2089.0 | 2118 | 46 |
| Run #1 | Ken |  | Test Data |  |  |  |  |  |  | Run #2 | Ant |  | Test Data |  |  |  |
| MP | RPM | IAS | PA | OAT | Time | HP | BR | GW | MP | RPM | IAS | PA | OAT | TIme | HP | GW |
| 22.7 | 2700 | 129 | 7500 | 60 | 0.0 | 142 | 11.5 | 2147.4 | 22.7 | 2700 | 134 | 7500 | 60 | 10.0 | 142 | 2109.4 |
| 22.7 | 2600 | 130 | 7500 | 60 | 3.0 | 139 | 11.5 | 2143.5 | 22.7 | 2600 | 131 | 7500 | 60 | 1.0 | 139 | 2108.1 |
| 22.7 | 2500 | 126 | 7500 | 60 | 3.0 | 134 | 8.8 | 2140.5 | 22.7 | 2500 | 126 | 7500 | 60 | 2.0 | 134 | 2106.1 |
| 22.7 | 2450 | 124 | 7500 | 60 | 1.0 | 132 | 8.8 | 2139.5 | 22.7 | 2450 | 125 | 7500 | 60 | 1.0 | 132 | 2105.1 |
| 22.7 | 2400 | 123 | 7500 | 60 | 1.0 | 131 | 8.8 | 2138.5 | 22.7 | 2400 | 124 | 7520 | 60 | 1.0 | 131 | 2104.1 |
| 22.7 | 2300 | 121 | 7500 | 60 | 1.0 | 126 | 8.8 | 2137.5 | 22.7 | 2300 | 123 | 7500 | 60 | 1.0 | 126 | 2103.1 |
| 22.5 | 2250 | 120 | 7500 | 60 | 3.0 | 124 | 8.8 | 2134.5 | 22.5 | 2250 | 120 | 7500 | 60 | 2.0 | 124 | 2101.1 |
| 22 | 2200 | 117 | 7500 | 60 | 2.0 | 118 | 7.5 | 2132.8 | 22 | 2200 | 116 | 7500 | 60 | 1.0 | 118 | 2100.2 |
| 21.5 | 2200 | 115 | 7500 | 60 | 1.0 | 113 | 7.5 | 2131.9 | 21.5 | 2200 | 114 | 7500 | 60 | 1.0 | 113 | 2099.4 |
| 21 | 2200 | 109 | 7500 | 60 | 1.0 | 108 | 7.5 | 2131.1 | 21 | 2200 | 110 | 7520 | 60 | 2.0 | 108 | 2097.7 |
| 20 | 2200 | 103 | 7500 | 60 | 1.0 | 102 | 7.5 | 2130.2 | 20 | 2200 | 104 | 7500 | 60 | 2.0 | 102 | 2096.0 |
| 19 | 2200 | 99 | 7500 | 60 | 1.0 | 95 | 6.3 | 2129.5 | 19 | 2200 | 85 | 7520 | 60 | 3.0 | 95 | 2093.9 |
| 18.5 | 2200 | 90 | 7500 | 60 | 1.0 | 92 | 6.3 | 2128.8 | 18.5 | 2200 | 80 | 7540 | 60 | 1.0 | 92 | 2093.1 |
| 19.5 | 2200 | 85 | 7480 | 60 | 1.0 | 99 | 6.3 | 2128.1 | 19 | 2200 | 76 | 7500 | 60 | 2.0 | 95 | 2091.7 |
| 19 | 2200 | 80 | 7500 | 60 | 1.0 | 95 | 6.3 | 2127.4 | 19.5 | 2200 | 70 | 7400 | 60 | 2.0 | 99 | 2090.3 |
| 18.5 | 2200 | 75 | 7500 | 60 | 1.0 | 92 | 6.3 | 2126.7 | 20.5 | 2200 | 65 | 7500 | 60 | 4.0 | 105 | 2087.4 |
| 21 | 2200 | 70 | 7500 | 60 | 5.0 | 108 | 7.5 | 2122.4 |  |  |  |  |  |  |  |  |

# Appendix B

Table 15 Instrument Corrections for 7500 ft

| 7500ft | rho std | sigstd | gama | ao |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0.0019 | 0.80042 | 1.4 | 1116.288 |  |  |  |  |
| Ave IAS | Ave Alt | Delt Vpc | delt Hpc | Hc | Ts | Ps | rho | rhostd |
| 131.50 | 7500.00 | -1.61 | -17.74 | 7482.26 | 31.14 | 1603.68 | 0.001794 | 0.001899 |
| 130.50 | 7500.00 | -1.53 | -16.80 | 7483.20 | 31.14 | 1603.62 | 0.001794 | 0.001898 |
| 126.00 | 7500.00 | -1.17 | -12.34 | 7487.66 | 31.12 | 1603.35 | 0.001794 | 0.001898 |
| 124.50 | 7500.00 | -1.03 | -10.78 | 7489.22 | 31.12 | 1603.26 | 0.001794 | 0.001898 |
| 123.50 | 7510.00 | -0.94 | -9.72 | 7500.28 | 31.08 | 1602.58 | 0.001793 | 0.001897 |
| 122.00 | 7500.00 | -0.79 | -8.11 | 7491.89 | 31.11 | 1603.09 | 0.001794 | 0.001898 |
| 120.00 | 7500.00 | -0.59 | -5.92 | 7494.08 | 31.10 | 1602.96 | 0.001793 | 0.001898 |
| 116.50 | 7500.00 | -0.21 | -2.02 | 7497.98 | 31.09 | 1602.72 | 0.001793 | 0.001898 |
| 114.50 | 7500.00 | 0.02 | 0.24 | 7500.24 | 31.08 | 1602.59 | 0.001793 | 0.001897 |
| 109.50 | 7510.00 | 0.65 | 5.95 | 7515.95 | 31.02 | 1601.63 | 0.001792 | 0.001897 |
| 103.50 | 7500.00 | 1.49 | 12.88 | 7512.88 | 31.03 | 1601.81 | 0.001792 | 0.001897 |
| 99.00 | 7510.00 | 2.19 | 18.13 | 7528.13 | 30.98 | 1600.88 | 0.001791 | 0.001896 |
| 90.00 | 7520.00 | 3.85 | 29.02 | 7549.02 | 30.91 | 1599.61 | 0.00179 | 0.001895 |
| 85.00 | 7490.00 | 4.99 | 35.52 | 7525.52 | 30.99 | 1601.04 | 0.001791 | 0.001896 |
| 78.00 | 7450.00 | 6.99 | 45.59 | 7495.59 | 31.10 | 1602.87 | 0.001793 | 0.001898 |
| 75.00 | 7500.00 | 8.03 | 50.37 | 7550.37 | 30.90 | 1599.53 | 0.00179 | 0.001895 |
| 70.00 | 7500.00 | 10.09 | 59.09 | 7559.09 | 30.87 | 1599.00 | 0.001789 | 0.001894 |
|  |  |  |  |  | Interpolation Values for 7000 ft |  |  |  |
|  |  |  |  |  | A | Ts | Ps | rhostd |
|  |  |  |  |  | 7000 | 493.73 | 1633.1 | 0.00193 |
|  |  |  |  |  | 8000 | 490.17 | 1572.1 | 0.00187 |

Figure 15 Prop efficiency

AA4323 Class Notes

AA4323 Flight Test Engineering Class Notes

Anderson, “Introduction To Flight”, Ch.6

AA4323 Flight Test Engineering Class Notes

AA4323 Flight Test Engineering Class Notes